Ear Dimensions & Weight HIPS 2022

- Turn on the scale by pressing the Zero button on the scale

- Open programs (located on the desktop) to run in the background as you collect data

  - Winwedge – OHAUS_Scale_Carriage_Return_HIPS

  - Vmux LITE

    - The program needs a moment to recognize the dongle, but you can work on below while it ‘finds it’

    - Open / expand the settings tab on the right

    - Select export to bring up a new window

    - In that window select Export mode “In focused window”

    - And change the End char is {TAB}

    - Select OK when done to close that export window

    - Close the settings tab

- From Box, open up your Excel file:  3_Ear_Traits_TEMPLATE.xlsx

- “Save As” & replace TEMPLATE with date & initials

  - example 	3_Ear_Traits 11.04.22_lc

- Collect Data

  - During you shift

    - Save periodically

    - Make sure that the

      - Scale

        - The scale is in g

        - The bucket on the scale is tared out (ie the scale reads 0 even though the bucket is on the scale

        - If you need to tare out the bucket weight, just press and hold the Tare button

      - Caliper

        - The caliper is in mm

        - The caliper reads as 0 when the slider is closed

        - The Bluetooth icon is showing on the top right of the digital screen of the caliper

        - See section below about the Caliper if you need help with the above bullet points

  - Scan your ear barcode

  - Collect your ear length

  - If your cob is curved

    - use string to measure to run the length of the cob

    - lay the string down on the table and use the caliper to measure the points you marked

    - note on the file that you used the string to obtain width

  - Collect your ear width at the widest part of the ear – typically the lower 1/3 of your ear

  - Place the ear in the bucket on the scale and press “Print”

  - The devices & scanner are programmed to move to where you need them next

    - The scanner is programmed to tab over after scanning

    - The Vmux LITE is programmed to tab over for you transmit data

    - The Scale’s winwedge is programmed to carriage return to the 1st column and the next Excel row below ie ready for you to scan the next barcode

- End of shift

  - Turn off scale by press and holding the Zero button

  - Caliper

    - Close the caliper & tighten the locking screw at the top

    - Press & hold the set button for > 6 seconds

  - Save and close the data collection file

  - Close the Vmux LITE & Windwedge programs

  - Log out of Box

(Below was copied from Word Doc Caliper_Sylvac_Bluetooth)

Caliper Sylvac Bluetooth

Short press    vs     Hold press

Turn on:

- Unlock screw at the top and move the measure slider

Turn off:

- Close the slider

- Tighten the locking screw on top (this will help to avoid accidental turn on and battery drain)

- Press & hold set button for >6 seconds

***Remember, if you get in the settings and need out, press the data button on top right to exit out

Start of shift:

- Make sure units are in mm (see section below about how to change if needed)

- Make sure caliper measures 0 when slider is closed

- Make sure the Bluetooth icon is showing on the top right of digital screen of the caliper

  - If it doesn’t show, you need to go into settings to turn it on (see next)

  - Turn on Bluetooth:  Press mode 3x so it says ‘bt’, press set to toggle until it says ‘on’

How to reset 0

- Close the slider

- Press mode, Press set (both short presses)

How to change units to mm

- Press & hold mode for over 1 second, screen will show ‘Unit’

  - If you accidentally press mode again, it will move to Bluetooth Configurtion (bt.CFG) settings.

  - Just press mode again to toggle back to ‘unit’.  Or wait for it to time out (few seconds) or press the data button to exit back out

- When screen says ‘Unit’, press Set

- Press Set to toggle between mm and inches

- Either wait a few seconds, or press the data button to exit out